MASINDE MULIRO UNIVERSITY OF SCIENCE AND TECHNOLOGY

DEPARTMENT OF CIVIL AND STRUCTURAL ENGINEERING

CSE 215: ENGINEERING DRAWING 3

TYPES OF BUILDING DRAWINGS

Plan Drawing

A plan drawing shows an area of a building seen from the top of horizontal plane. There are four types of plans, namely

1.Location plan

2.Site plan

3.Floor plan

4.Roof plan

1. Location Plan

- Shows the location of a proposed construction site.

- In this plan several information related to construction site are stated such as street names, lot numbers, adjacent buildings, main road and compass direction.

- Usually drawn without scale

2. Site Plan

- A view looking down at a building from above, illustrating its location and orientation on a plot land and providing information about the site’s topography, landscape and site work.

- It illustrates the existing natural and built features.

- The information included in site plan are the lot numbers, construction site dimensions, location of proposed construction site, entry road, septic tank and nearby areas.

- Can help contractors to determine the appropriate working area, construction material storage, workers quarters and site office.

- Site plan usually drawn using scale 1:200.

2. Floor Plan

A floor plan is sectional view of a building through a horizontal cutting plane, at about 1.5m from the floor.

For multi-storey buildings, each storey has its own floor plan.

A floor plan shows the section in a building and standard symbols for windows, doors, walls and other details.

The scales usually used in floor plans are 1:50, 1:100 and 1:200.

Functions:

- Defines spaces and their functional relationships

- Illustrates openings (doors and windows)

- Finishing

- Entrance (main, secondary)

- Utilities (stairs, elevators, baths, stores….)

- Structural systems – Reinforced concrete, columns , slabs, beams, walls

- Measurements

4. Roof Plan

Roof plan describes the shape of roof, its dimension and inclination viewed from the top view.

Roof components such as ridge, valley, hip. Gutter and roof tiles are shown in roof plan.

The normally used scales in roof plan are 1:50, 1:100 and 1:200

VIEW DRAWING

The drawing of a building exterior viewed from the front, back, right side and left side.

Elevation view is seen horizontally in the form of an orthographic drawing.

Details in an elevation include columns, walls, doors, windows and roof.

The normally used scales are 1:50, 1:100 and 1:200.

SECTIONAL DRAWING

Shows the cross-sections of a building and other details.

A vertical cutting plane cuts top of the building to the ground through the roof, columns, floor, footing, windows and doors.

Based on the floor plan using standard symbols.

The information in a sectional drawing includes height, component sizes, and construction materials specification.

- SITE PLAN

Building blocks:

- Property lines

- Outline of existing and proposed buildings and structures

- Distance between buildings

- Distance between buildings and property lines (setbacks)

- Parking lots, indicating parking spaces

- Driveways

- Surrounding streets

- Landscaped areas

- Easements

- Ground sign location

- Utilities

Site plans shall be coloured as follows:

- Proposed work                                                      Red

- Existing work                                                       Not coloured

- Work to be demolished                                        Drawn with black dotted lines AA 11